|  | #### Job Safety Analysis |  |  |
| --- | --- | --- | --- |
|  | Portland State University |  |  |
|  | Facilities Services |  |  |
|  | ## Using a Pipe Threader |  |  |
|  | Task | Hazards | Controls |
|  | - Check cutting oil and clean the catch pan. | Slips | Control oil contact with the floor. Wear non-slip shoes. Clean up spills. Use a floor covering or absorbent such as kitty litter. |
|  |  | Skin or eye injury from the cutting oil in the reservoir | Use nitrile gloves. Use a vacuum cleaner to clean the catch pan. Wear eye protection. |
|  | 2. Feed the pipe into the sleeve. | Hand injury while sliding the pipe through. | Wear leather gloves or similar protection. |
|  |  | Muscle strain | Position pipe without reaching over the threader. |
|  | 3. Clamp the pipe in place. | Hand injury | Wear leather gloves or similar protection. Keep your foot out of the foot safety switch. Don’t tighten the clamp while the machine is running. |
|  | 4. Ream pipe. | Hand injury | Don’t drop the reamer on your hand while positioning the reamer. |
|  | 5. Load the appropriate cutting die. | Hand injury | Keep hand away from the die. |
|  |  |  | Don’t clean the die or the pipe end with your hands. Don’t start the machine until the die is set. |
|  | 6. Start the threader. | Injuries due to clothes catching on the threader | Don’t wear loose clothing while operating the threader. Keep your gloved hands away from the die and the pipe. |
|  |  | Skin or eye injury from the cutting oil being delivered to the pipe. | Use nitrile gloves. Wear eye protection. |
|  | 7. Feed the die. | Metal debris projectiles | Wear long sleeved shirt, gloves, and face |
|  |  | Burns from metal debris | Wear long sleeved shirt, gloves, and face shield. |
|  | 8. Release the die. | None foreseen |  |
|  | 9. Retract the die. | None foreseen |  |
|  | 10. Wipe excess oil and metal debris from the pipe. | Hand injury from rotating threads | Wear gloves and don’t touch the rotating threads. Use a steel brush or similar instrument to clean the threads. |
|  |  | Injuries due to clothing catching on the threader | Don’t wear loose clothing while operating the threader. Keep your gloved hands away from the rotating pipe. |
|  | 11. Loosen and pull the pipe. | Hand injuries from interaction with the pipe vise. | Remove foot from the safety switch, flip the switch off, and ensure the pipe has stopped rotating. Wear gloves. |
|  |  | Hand injuries from interaction with the pipe vise | Remove foot from the safety switch, flip the switch off, and ensure the pipe has stopped rotating. Keep your fingers (gloved) away from the vise jaws. |
|  | Required Training: - Training in the operation of the pipe vise - Proper lifting techniques | Required Personal Protective Equipment (PPE) - Nitrile gloves - Leather (or similar gloves) - Eye protection - Face shield - Long sleeved shirt |  |
| Other Information: |  |  |  |
| Contributors: | Reviewed JKM |  |  |
| Created: | July 2006 |  |  |